COMPUTER NETWORKS

Comprehensive Exam Preparation

200 Multiple-Choice Questions with Answers & Explanations

Topics: OSI/TCP-IP · Routing · Subnetting · NAT · DHCP · DNS · VPN · QoS · 802.11 · Security · IPv6 · WAN · Cloud · Protocols · Performance

# Section 1: OSI & TCP/IP Models

Q1. How many layers does the OSI model have?

A) 4

B) 5

C) 7

D) 6

✔ Correct Answer: C) 7

Explanation: The OSI (Open Systems Interconnection) model defines seven layers: Physical, Data Link, Network, Transport, Session, Presentation, and Application.

Q2. Which OSI layer is responsible for end-to-end error detection and flow control?

A) Network

B) Data Link

C) Transport

D) Session

✔ Correct Answer: C) Transport

Explanation: The Transport layer (Layer 4) ensures reliable end-to-end data transfer, handling segmentation, flow control, and error correction between hosts.

Q3. At which OSI layer does routing occur?

A) Layer 2 – Data Link

B) Layer 3 – Network

C) Layer 4 – Transport

D) Layer 1 – Physical

✔ Correct Answer: B) Layer 3 – Network

Explanation: The Network layer (Layer 3) handles logical addressing and routing, determining the best path for data to travel across networks using IP addresses.

Q4. Which TCP/IP model layer corresponds to both the Session and Presentation layers of the OSI model?

A) Internet

B) Transport

C) Application

D) Network Access

✔ Correct Answer: C) Application

Explanation: The TCP/IP Application layer encompasses the OSI Session, Presentation, and Application layers, handling high-level protocols, data representation, and session management.

Q5. What is the PDU (Protocol Data Unit) at the Data Link layer?

A) Segment

B) Packet

C) Frame

D) Bit

✔ Correct Answer: C) Frame

Explanation: The Data Link layer encapsulates data into frames, which include MAC addresses for local network delivery and error-checking via CRC.

Q6. Which layer of the OSI model converts data into a format suitable for the application layer?

A) Session

B) Presentation

C) Application

D) Transport

✔ Correct Answer: B) Presentation

Explanation: The Presentation layer handles data translation, encryption/decryption, and compression, ensuring data from the network is in a usable format for the application layer.

Q7. A network engineer captures traffic and observes MAC addresses being used for forwarding. Which OSI layer is operating?

A) Network

B) Physical

C) Transport

D) Data Link

✔ Correct Answer: D) Data Link

Explanation: The Data Link layer uses MAC (Media Access Control) addresses to identify devices on the same local network segment and frame delivery.

Q8. Which of the following best describes encapsulation in networking?

A) Removing headers as data travels down the stack

B) Adding headers/trailers as data moves down the protocol stack

C) Encrypting data at the transport layer

D) Converting analog signals to digital

✔ Correct Answer: B) Adding headers/trailers as data moves down the protocol stack

Explanation: Encapsulation adds protocol-specific headers (and trailers) at each layer as data travels from the application down to the physical layer before transmission.

Q9. The TCP/IP model's Internet layer maps to which OSI layer?

A) Data Link

B) Transport

C) Network

D) Session

✔ Correct Answer: C) Network

Explanation: The TCP/IP Internet layer performs the same functions as the OSI Network layer, specifically IP addressing and routing between networks.

Q10. Which protocol operates at the Application layer of the TCP/IP model and provides network management capabilities?

A) IP

B) SNMP

C) TCP

D) ARP

✔ Correct Answer: B) SNMP

Explanation: SNMP (Simple Network Management Protocol) operates at the Application layer and is used to monitor and manage network devices such as routers and switches.

# Section 2: Switching & Network Stacking

Q11. What does a Layer 2 switch use to forward frames?

A) IP addresses

B) MAC addresses

C) Port numbers

D) Domain names

✔ Correct Answer: B) MAC addresses

Explanation: Layer 2 switches build a MAC address table (CAM table) and use it to make forwarding decisions based on destination MAC addresses within a local network.

Q12. When a switch receives a frame for an unknown destination MAC address, what does it do?

A) Drops the frame

B) Sends it back to the source

C) Floods it out all ports except the source port

D) Forwards it to the default gateway

✔ Correct Answer: C) Floods it out all ports except the source port

Explanation: When a switch doesn't have a destination MAC address in its CAM table, it floods the frame out all ports except the incoming port, a process called unknown unicast flooding.

Q13. What is the purpose of the Spanning Tree Protocol (STP)?

A) To increase available bandwidth

B) To prevent Layer 2 loops in switched networks

C) To assign IP addresses dynamically

D) To enable routing between VLANs

✔ Correct Answer: B) To prevent Layer 2 loops in switched networks

Explanation: STP (IEEE 802.1D) eliminates loops in redundant switched networks by blocking redundant paths and activating them only when the primary path fails.

Q14. What is a VLAN primarily used for?

A) To increase physical distance coverage

B) To logically segment a network without additional physical hardware

C) To automatically assign IP addresses

D) To encrypt network traffic

✔ Correct Answer: B) To logically segment a network without additional physical hardware

Explanation: VLANs allow a single physical switch to be divided into multiple logical networks, improving security and reducing broadcast domains without extra hardware.

Q15. Which port type carries traffic for multiple VLANs between switches?

A) Access port

B) Trunk port

C) Uplink port

D) Mirror port

✔ Correct Answer: B) Trunk port

Explanation: Trunk ports carry tagged traffic for multiple VLANs using protocols like IEEE 802.1Q, enabling VLAN communication across switches.

Q16. In a stacked switch configuration, what is the primary benefit?

A) Each switch operates completely independently

B) Multiple switches appear and are managed as a single logical unit

C) Bandwidth is halved for redundancy

D) Only one switch is active at a time

✔ Correct Answer: B) Multiple switches appear and are managed as a single logical unit

Explanation: Switch stacking interconnects multiple physical switches to function as one logical device, simplifying management and expanding port density.

Q17. What is MAC flooding in the context of network security?

A) Sending too many multicast frames

B) Overflowing the switch's CAM table to force it into hub-like behavior

C) Assigning duplicate MAC addresses

D) Blocking all broadcast traffic

✔ Correct Answer: B) Overflowing the switch's CAM table to force it into hub-like behavior

Explanation: MAC flooding attacks overwhelm a switch's CAM table with fake MAC addresses, causing the switch to flood all frames out every port, enabling traffic interception.

Q18. Which IEEE standard defines the 802.1Q VLAN tagging protocol?

A) IEEE 802.3

B) IEEE 802.11

C) IEEE 802.1Q

D) IEEE 802.1X

✔ Correct Answer: C) IEEE 802.1Q

Explanation: IEEE 802.1Q defines the standard for VLAN tagging in Ethernet frames, inserting a 4-byte tag that identifies the VLAN and priority of each frame on trunk links.

# Section 3: Routing – Distance-Vector & Link-State

Q19. Which routing algorithm does RIP (Routing Information Protocol) use?

A) Link-state

B) Path-vector

C) Distance-vector

D) Hybrid

✔ Correct Answer: C) Distance-vector

Explanation: RIP uses the Bellman-Ford distance-vector algorithm, where each router shares its routing table with directly connected neighbors periodically.

Q20. What metric does RIP use to determine the best path?

A) Bandwidth

B) Hop count

C) Delay

D) Load

✔ Correct Answer: B) Hop count

Explanation: RIP uses hop count as its only metric, with a maximum of 15 hops (16 = unreachable), making it simple but unsuitable for large networks.

Q21. OSPF is an example of which type of routing protocol?

A) Distance-vector

B) Path-vector

C) Link-state

D) Hybrid

✔ Correct Answer: C) Link-state

Explanation: OSPF (Open Shortest Path First) is a link-state protocol where each router maintains a full topology map of the network (LSDB) and uses Dijkstra's algorithm to compute shortest paths.

Q22. What is the 'count to infinity' problem associated with?

A) Link-state protocols

B) Distance-vector protocols

C) BGP

D) Static routing

✔ Correct Answer: B) Distance-vector protocols

Explanation: Count to infinity occurs in distance-vector protocols when routing loops cause hop counts to increment continuously until the maximum value is reached, causing convergence delays.

Q23. Which technique helps distance-vector protocols avoid routing loops by not advertising a route back through the interface it was learned from?

A) Route poisoning

B) Split horizon

C) Hold-down timers

D) Triggered updates

✔ Correct Answer: B) Split horizon

Explanation: Split horizon prevents routing loops by prohibiting a router from advertising a route back out the same interface through which that route was learned.

Q24. In OSPF, what is the role of the Designated Router (DR)?

A) To block redundant paths

B) To reduce LSA flooding by acting as the central point for LSA exchange on a multi-access network

C) To assign IP addresses

D) To encrypt OSPF packets

✔ Correct Answer: B) To reduce LSA flooding by acting as the central point for LSA exchange on a multi-access network

Explanation: On multi-access networks (e.g., Ethernet), the DR collects and distributes LSAs to all OSPF routers, reducing the number of adjacencies and LSA traffic.

Q25. Which protocol is used for routing between autonomous systems on the Internet?

A) OSPF

B) RIP

C) EIGRP

D) BGP

✔ Correct Answer: D) BGP

Explanation: BGP (Border Gateway Protocol) is a path-vector protocol used as the standard inter-domain routing protocol connecting different Autonomous Systems (AS) on the Internet.

Q26. A router running EIGRP loses its primary route. What does it check before querying neighbors?

A) BGP table

B) The feasible successor in its topology table

C) ARP cache

D) DNS records

✔ Correct Answer: B) The feasible successor in its topology table

Explanation: EIGRP maintains a topology table with feasible successors (pre-calculated backup routes). If a feasible successor exists, it becomes the new successor without querying neighbors, enabling fast convergence.

Q27. What administrative distance does OSPF have by default on Cisco routers?

A) 90

B) 110

C) 120

D) 20

✔ Correct Answer: B) 110

Explanation: Cisco assigns OSPF an administrative distance of 110. Lower AD values are preferred; OSPF (110) is preferred over RIP (120) but less preferred than EIGRP internal routes (90).

Q28. Which routing protocol sends full routing table updates every 30 seconds by default?

A) OSPF

B) BGP

C) RIPv2

D) EIGRP

✔ Correct Answer: C) RIPv2

Explanation: RIPv2 broadcasts its entire routing table to neighbors every 30 seconds, which is inefficient for large networks compared to event-driven protocols like OSPF.

# Section 4: IP Addressing & Subnetting

Q29. What is the subnet mask for a /24 prefix in dotted-decimal notation?

A) 255.255.0.0

B) 255.255.255.0

C) 255.0.0.0

D) 255.255.255.128

✔ Correct Answer: B) 255.255.255.0

Explanation: A /24 prefix means 24 bits are the network portion. In dotted-decimal, 24 consecutive 1-bits translate to 255.255.255.0.

Q30. How many usable host addresses are available in a /26 subnet?

A) 62

B) 30

C) 126

D) 64

✔ Correct Answer: A) 62

Explanation: A /26 leaves 6 host bits, giving 2^6 = 64 total addresses. Subtracting the network and broadcast addresses leaves 62 usable host addresses.

Q31. Which address class was historically assigned to large organizations with networks up to 16 million hosts?

A) Class B

B) Class C

C) Class A

D) Class D

✔ Correct Answer: C) Class A

Explanation: Class A addresses (1.0.0.0–126.255.255.255) use only 8 bits for the network and 24 bits for hosts, supporting ~16.7 million hosts per network.

Q32. What is the broadcast address for the network 192.168.10.0/28?

A) 192.168.10.15

B) 192.168.10.31

C) 192.168.10.14

D) 192.168.10.255

✔ Correct Answer: A) 192.168.10.15

Explanation: A /28 mask means 4 host bits. The network 192.168.10.0/28 spans .0–.15. The broadcast is the last address: 192.168.10.15.

Q33. Which private IP range is defined by RFC 1918 for Class B addresses?

A) 10.0.0.0/8

B) 172.16.0.0/12

C) 192.168.0.0/16

D) 169.254.0.0/16

✔ Correct Answer: B) 172.16.0.0/12

Explanation: RFC 1918 defines three private ranges: 10.0.0.0/8 (Class A), 172.16.0.0–172.31.255.255/12 (Class B), and 192.168.0.0/16 (Class C).

Q34. VLSM (Variable Length Subnet Masking) allows network administrators to:

A) Use only classful subnet masks

B) Assign subnet masks of different lengths to different subnets

C) Automatically assign IP addresses

D) Eliminate the need for a default gateway

✔ Correct Answer: B) Assign subnet masks of different lengths to different subnets

Explanation: VLSM enables efficient IP address utilization by allowing different subnets within the same network to use different prefix lengths based on their host count requirements.

Q35. What is the network address of the IP 10.5.44.61/21?

A) 10.5.40.0

B) 10.5.44.0

C) 10.5.32.0

D) 10.5.48.0

✔ Correct Answer: A) 10.5.40.0

Explanation: A /21 mask is 255.255.248.0. 44 in binary is 00101100. ANDing with 11111000 gives 00101000 = 40. So the network is 10.5.40.0.

Q36. Which IP address is reserved for loopback testing?

A) 0.0.0.0

B) 255.255.255.255

C) 127.0.0.1

D) 169.254.0.1

✔ Correct Answer: C) 127.0.0.1

Explanation: The 127.0.0.0/8 range (commonly 127.0.0.1) is reserved for loopback; packets sent to this address are looped back to the sending device for testing network stack.

Q37. How many subnets can be created from a /24 network if you subnet it into /27 networks?

A) 4

B) 6

C) 8

D) 16

✔ Correct Answer: C) 8

Explanation: Moving from /24 to /27 borrows 3 bits (27–24=3). 2^3 = 8 subnets, each with 30 usable hosts.

Q38. What is CIDR (Classless Inter-Domain Routing) primarily designed to address?

A) MAC address conflicts

B) IPv4 address exhaustion and routing table growth

C) Wireless interference

D) DNS resolution failures

✔ Correct Answer: B) IPv4 address exhaustion and routing table growth

Explanation: CIDR replaced classful addressing to allow flexible prefix lengths, enabling route aggregation (supernetting) which slows routing table growth and extends IPv4 address space.

Q39. A host is assigned the APIPA address 169.254.45.12. What does this indicate?

A) The host has a static IP configured

B) The host failed to obtain an IP address from a DHCP server

C) The host is using IPv6

D) The host is behind a NAT device

✔ Correct Answer: B) The host failed to obtain an IP address from a DHCP server

Explanation: APIPA (Automatic Private IP Addressing) assigns a 169.254.0.0/16 address when a DHCP server is unreachable, allowing limited local-only communication.

Q40. Given the IP address 192.168.1.130/25, which subnet does this host belong to?

A) 192.168.1.0/25

B) 192.168.1.128/25

C) 192.168.1.64/25

D) 192.168.1.192/25

✔ Correct Answer: B) 192.168.1.128/25

Explanation: A /25 splits the last octet at bit 7. 130 > 128, so the host falls in the 192.168.1.128/25 subnet (range: 192.168.1.128–192.168.1.255).

# Section 5: NAT (Network Address Translation)

Q41. What is the primary purpose of NAT?

A) To encrypt data between hosts

B) To translate private IP addresses to public IP addresses for Internet access

C) To assign IP addresses automatically

D) To resolve domain names to IP addresses

✔ Correct Answer: B) To translate private IP addresses to public IP addresses for Internet access

Explanation: NAT allows multiple devices with private (non-routable) IP addresses to share one or more public IP addresses when communicating over the Internet.

Q42. Which NAT type uses a pool of public IP addresses mapped one-to-one with private addresses?

A) PAT (Port Address Translation)

B) Static NAT

C) Dynamic NAT

D) NAT64

✔ Correct Answer: C) Dynamic NAT

Explanation: Dynamic NAT maps private IP addresses to a pool of public IP addresses on a first-come, first-served basis, creating temporary one-to-one mappings.

Q43. PAT (Port Address Translation) is also known as:

A) Static NAT

B) NAT overload

C) Double NAT

D) Bi-directional NAT

✔ Correct Answer: B) NAT overload

Explanation: PAT, or NAT overload, maps multiple private IP addresses to a single public IP by differentiating connections using unique source port numbers.

Q44. Which NAT table entry type is permanently configured regardless of traffic?

A) Dynamic NAT

B) PAT

C) Static NAT

D) Reflexive NAT

✔ Correct Answer: C) Static NAT

Explanation: Static NAT creates a permanent, fixed one-to-one mapping between a specific private IP and a specific public IP, commonly used to make internal servers accessible from the Internet.

Q45. A company uses PAT with one public IP. The NAT table entry uniquely identifies each session using:

A) Destination IP and protocol

B) Source IP, source port, destination IP, destination port, and protocol

C) Only the source IP address

D) MAC addresses of communicating hosts

✔ Correct Answer: B) Source IP, source port, destination IP, destination port, and protocol

Explanation: PAT distinguishes simultaneous sessions from different internal hosts by creating unique combinations of source IP, source port, destination IP, destination port, and protocol.

Q46. What is a limitation of NAT in modern networking?

A) It increases routing table size

B) It breaks end-to-end connectivity required by some protocols like IPSec and SIP

C) It prevents DHCP from functioning

D) It makes the network slower by 50%

✔ Correct Answer: B) It breaks end-to-end connectivity required by some protocols like IPSec and SIP

Explanation: NAT modifies packet headers, breaking protocols that embed IP addresses in payloads (like SIP) or require unmodified headers for authentication (like IPSec AH).

Q47. NAT64 is used to facilitate communication between:

A) Two IPv4 hosts on different networks

B) IPv6-only clients and IPv4-only servers

C) Two OSPF autonomous systems

D) Wireless and wired networks

✔ Correct Answer: B) IPv6-only clients and IPv4-only servers

Explanation: NAT64 is a transition mechanism that translates IPv6 packets to IPv4 packets (and vice versa), enabling IPv6-only devices to communicate with IPv4-only services.

Q48. In NAT terminology, 'inside global' refers to:

A) The private IP address of an internal host

B) The public IP address representing an internal host to the outside world

C) The IP address of the NAT router's LAN interface

D) Any RFC 1918 address

✔ Correct Answer: B) The public IP address representing an internal host to the outside world

Explanation: Inside global is the translated (public) IP address that represents an internal private host when communicating across the Internet.

# Section 6: DHCP (Dynamic Host Configuration Protocol)

Q49. What is the DHCP DORA process in the correct order?

A) Discover, Offer, Request, Acknowledge

B) Discover, Request, Offer, Acknowledge

C) Offer, Discover, Request, Acknowledge

D) Request, Offer, Discover, Acknowledge

✔ Correct Answer: A) Discover, Offer, Request, Acknowledge

Explanation: DORA: The client broadcasts a Discover, the server responds with an Offer, the client broadcasts a Request for that offer, and the server sends an Acknowledgment confirming the lease.

Q50. On which port does a DHCP server listen for client requests?

A) UDP 67

B) UDP 68

C) TCP 67

D) TCP 80

✔ Correct Answer: A) UDP 67

Explanation: DHCP servers listen on UDP port 67, while DHCP clients receive responses on UDP port 68. DHCP uses UDP because it operates before a host has a configured IP address.

Q51. What is a DHCP lease?

A) A permanent IP assignment

B) A temporary IP address assignment for a defined period

C) An encryption key for DHCP messages

D) A backup IP for failover

✔ Correct Answer: B) A temporary IP address assignment for a defined period

Explanation: A DHCP lease is a time-limited assignment of an IP address to a client. When the lease expires, the client must renew or request a new address.

Q52. What is the purpose of a DHCP relay agent (IP helper)?

A) To encrypt DHCP packets

B) To forward DHCP broadcasts between subnets to a central DHCP server

C) To assign static IPs based on hostname

D) To block rogue DHCP servers

✔ Correct Answer: B) To forward DHCP broadcasts between subnets to a central DHCP server

Explanation: A DHCP relay agent (configured with 'ip helper-address') forwards DHCP broadcasts as unicast to a DHCP server on a different subnet, enabling one server to serve multiple subnets.

Q53. Which DHCP option provides the IP address of the default gateway to clients?

A) Option 3

B) Option 6

C) Option 15

D) Option 51

✔ Correct Answer: A) Option 3

Explanation: DHCP Option 3 specifies the default gateway (router) for the client. Option 6 provides DNS server addresses, and Option 51 specifies the lease time.

Q54. A DHCP client renews its lease at what point in the lease duration by default?

A) 50% (T1)

B) 75% (T2)

C) 25% (T1)

D) 90% (T1)

✔ Correct Answer: A) 50% (T1)

Explanation: By default, a DHCP client attempts to renew its lease at 50% of the lease duration (T1). If unsuccessful, it tries again at 87.5% (T2) before the lease expires.

Q55. What is a DHCP starvation attack?

A) Intercepting DHCP responses to provide false gateway information

B) Flooding a DHCP server with requests using spoofed MAC addresses to exhaust its address pool

C) Blocking DHCP packets at the firewall

D) Configuring a rogue DHCP server

✔ Correct Answer: B) Flooding a DHCP server with requests using spoofed MAC addresses to exhaust its address pool

Explanation: DHCP starvation attacks use spoofed MAC addresses to request all available IP leases, exhausting the pool and potentially performing a DoS against legitimate clients.

Q56. DHCP snooping is a security feature that:

A) Encrypts all DHCP traffic

B) Validates DHCP messages and drops those from untrusted sources

C) Automatically renews DHCP leases

D) Translates DHCP to BOOTP

✔ Correct Answer: B) Validates DHCP messages and drops those from untrusted sources

Explanation: DHCP snooping, configured on switches, classifies ports as trusted or untrusted. It drops DHCP server messages (OFFER, ACK) from untrusted ports to prevent rogue DHCP servers.

# Section 7: DNS (Domain Name System)

Q57. What is the primary function of DNS?

A) To assign IP addresses to hosts

B) To translate domain names to IP addresses

C) To route packets across the Internet

D) To encrypt web traffic

✔ Correct Answer: B) To translate domain names to IP addresses

Explanation: DNS translates human-readable domain names (like www.example.com) into IP addresses that computers use to identify each other on a network.

Q58. Which DNS record type maps a domain name to an IPv4 address?

A) AAAA

B) MX

C) A

D) CNAME

✔ Correct Answer: C) A

Explanation: A (Address) records map a hostname to an IPv4 address. AAAA records map to IPv6 addresses, MX records specify mail servers, and CNAME records create aliases.

Q59. On which port does DNS primarily operate?

A) TCP/UDP 53

B) TCP 80

C) UDP 161

D) TCP 443

✔ Correct Answer: A) TCP/UDP 53

Explanation: DNS uses UDP port 53 for standard queries (fast, low overhead). It switches to TCP port 53 for large responses (>512 bytes) and zone transfers.

Q60. What is a DNS zone transfer used for?

A) To encrypt DNS queries

B) To replicate DNS zone data from a primary DNS server to secondary servers

C) To block malicious domains

D) To increase DNS query speed

✔ Correct Answer: B) To replicate DNS zone data from a primary DNS server to secondary servers

Explanation: Zone transfers (AXFR/IXFR) allow secondary DNS servers to synchronize DNS zone records from the primary server for redundancy and load distribution.

Q61. What is the role of a DNS recursive resolver?

A) To store the root zone

B) To query other DNS servers on behalf of a client until it finds the answer

C) To assign domain names

D) To block spam domains

✔ Correct Answer: B) To query other DNS servers on behalf of a client until it finds the answer

Explanation: A recursive resolver accepts queries from clients and performs the full resolution process—querying root, TLD, and authoritative servers—then returns the final answer to the client.

Q62. Which type of DNS attack poisons the cache of a resolver with false records?

A) DNS amplification

B) DNS cache poisoning

C) DNS tunneling

D) BGP hijacking

✔ Correct Answer: B) DNS cache poisoning

Explanation: DNS cache poisoning (DNS spoofing) injects fraudulent DNS records into a resolver's cache, redirecting users to attacker-controlled servers instead of legitimate sites.

Q63. A DNS PTR record is used for:

A) Mapping a hostname to a mail server

B) Reverse DNS lookup — mapping an IP address back to a hostname

C) Creating a canonical name alias

D) Specifying DNS server location

✔ Correct Answer: B) Reverse DNS lookup — mapping an IP address back to a hostname

Explanation: PTR (Pointer) records enable reverse DNS lookups, allowing network tools to resolve an IP address back to its associated hostname, useful for logging and diagnostics.

Q64. DNSSEC provides security by:

A) Encrypting DNS query content end-to-end

B) Adding digital signatures to DNS records to verify their authenticity

C) Blocking all non-HTTPS traffic

D) Using a VPN for DNS queries

✔ Correct Answer: B) Adding digital signatures to DNS records to verify their authenticity

Explanation: DNSSEC adds cryptographic signatures (RRSIG records) to DNS responses, allowing resolvers to verify that records are genuine and haven't been tampered with.

# Section 8: Encryption & Certificate Authorities (CA)

Q65. What does asymmetric encryption use?

A) The same key for encryption and decryption

B) A pair of mathematically related public and private keys

C) A shared secret derived from a password

D) XOR operations on data blocks

✔ Correct Answer: B) A pair of mathematically related public and private keys

Explanation: Asymmetric encryption uses a key pair: data encrypted with the public key can only be decrypted with the corresponding private key, and vice versa.

Q66. What is the role of a Certificate Authority (CA)?

A) To route HTTPS traffic

B) To issue and sign digital certificates, vouching for the identity of the certificate holder

C) To generate encryption keys for end users

D) To block expired SSL certificates

✔ Correct Answer: B) To issue and sign digital certificates, vouching for the identity of the certificate holder

Explanation: A CA is a trusted entity that issues digital certificates binding a public key to an identity, enabling others to verify that a public key genuinely belongs to its claimed owner.

Q67. Which protocol is used to check if a digital certificate has been revoked in real time?

A) CRL

B) OCSP

C) PKI

D) LDAP

✔ Correct Answer: B) OCSP

Explanation: OCSP (Online Certificate Status Protocol) allows clients to query a CA's responder in real time to check the revocation status of a specific certificate, more efficient than downloading a full CRL.

Q68. In TLS handshake, what is the purpose of the 'Client Hello' message?

A) To send the client's certificate

B) To initiate the handshake, providing supported cipher suites and TLS version

C) To exchange symmetric keys

D) To terminate the TLS session

✔ Correct Answer: B) To initiate the handshake, providing supported cipher suites and TLS version

Explanation: The Client Hello message starts the TLS handshake, conveying the client's supported TLS versions, cipher suites, and a random value used in key derivation.

Q69. Which encryption algorithm is considered symmetric?

A) RSA

B) ECC

C) AES

D) Diffie-Hellman

✔ Correct Answer: C) AES

Explanation: AES (Advanced Encryption Standard) is a symmetric block cipher using the same key for both encryption and decryption, making it fast for bulk data encryption.

Q70. What is a self-signed certificate?

A) A certificate signed by a trusted root CA

B) A certificate signed by its own private key rather than a CA

C) A certificate with an infinite validity period

D) A certificate used only for code signing

✔ Correct Answer: B) A certificate signed by its own private key rather than a CA

Explanation: Self-signed certificates are signed by the entity itself rather than a trusted CA, so they are not inherently trusted by browsers and require manual trust configuration.

Q71. The Diffie-Hellman key exchange is used to:

A) Sign digital certificates

B) Securely establish a shared secret over an insecure channel without transmitting the key

C) Verify website identity

D) Compress data for transmission

✔ Correct Answer: B) Securely establish a shared secret over an insecure channel without transmitting the key

Explanation: Diffie-Hellman allows two parties to independently compute the same shared secret using exchanged public values, without ever transmitting the secret itself.

Q72. What does 'forward secrecy' (perfect forward secrecy) mean in TLS?

A) All future traffic is pre-encrypted

B) Past session keys cannot be compromised even if the server's long-term private key is exposed later

C) The server's certificate automatically renews

D) Data is encrypted twice for extra security

✔ Correct Answer: B) Past session keys cannot be compromised even if the server's long-term private key is exposed later

Explanation: PFS uses ephemeral key exchanges (like ECDHE) so each session uses a unique key. Compromising the server's private key doesn't allow decryption of previously captured sessions.

# Section 9: VPNs (Virtual Private Networks)

Q73. What is a VPN primarily used for?

A) To increase internet bandwidth

B) To create an encrypted, secure tunnel over a public network

C) To replace DNS servers

D) To block malware on endpoints

✔ Correct Answer: B) To create an encrypted, secure tunnel over a public network

Explanation: A VPN establishes an encrypted tunnel over the Internet or other public networks, allowing secure communication as if devices were on the same private network.

Q74. Which VPN protocol provides both authentication and encryption and operates at the Network layer?

A) PPTP

B) L2TP

C) IPSec

D) SSL

✔ Correct Answer: C) IPSec

Explanation: IPSec is a Layer 3 protocol suite that provides authentication (AH) and encryption (ESP) for IP packets, commonly used for site-to-site and remote-access VPNs.

Q75. What is the difference between IPSec tunnel mode and transport mode?

A) Tunnel mode encrypts only the payload; transport mode encrypts the entire packet

B) Tunnel mode encapsulates the entire IP packet; transport mode encrypts only the payload

C) Tunnel mode uses UDP; transport mode uses TCP

D) There is no functional difference

✔ Correct Answer: B) Tunnel mode encapsulates the entire IP packet; transport mode encrypts only the payload

Explanation: IPSec tunnel mode wraps the entire original IP packet (including headers) in a new packet—used for site-to-site VPNs. Transport mode only encrypts the payload—used for host-to-host communication.

Q76. SSL/TLS VPNs operate at which layer?

A) Layer 2

B) Layer 3

C) Layer 4/Application (Layer 7)

D) Layer 1

✔ Correct Answer: C) Layer 4/Application (Layer 7)

Explanation: SSL/TLS VPNs operate above Layer 4, enabling secure remote access through web browsers without requiring dedicated client software, unlike IPSec-based VPNs.

Q77. What protocol is commonly used in modern site-to-site VPNs for key management and SA negotiation?

A) L2TP

B) PPTP

C) IKE (Internet Key Exchange)

D) GRE

✔ Correct Answer: C) IKE (Internet Key Exchange)

Explanation: IKE (IKEv1/IKEv2) is the protocol used by IPSec to authenticate peers and negotiate Security Associations (SAs) including encryption algorithms and keys.

Q78. A split-tunnel VPN configuration means:

A) All traffic (internet and corporate) goes through the VPN tunnel

B) Only corporate-bound traffic goes through the VPN; internet traffic goes directly

C) The VPN splits bandwidth equally

D) Two separate VPN connections are maintained

✔ Correct Answer: B) Only corporate-bound traffic goes through the VPN; internet traffic goes directly

Explanation: Split tunneling routes only designated traffic (e.g., to corporate resources) through the VPN, while internet-bound traffic uses the client's local connection, reducing VPN load.

Q79. GRE (Generic Routing Encapsulation) tunnels are typically combined with IPSec because:

A) GRE provides encryption; IPSec provides tunneling

B) GRE provides the tunnel for multicast/routing protocols; IPSec provides the encryption

C) IPSec cannot work without GRE

D) GRE replaces all VPN protocols

✔ Correct Answer: B) GRE provides the tunnel for multicast/routing protocols; IPSec provides the encryption

Explanation: GRE alone lacks encryption but supports multicast and routing protocols. Combining GRE with IPSec (GRE over IPSec) provides a secure, encrypted tunnel that also supports routing protocol updates.

Q80. Which VPN technology is most commonly associated with modern remote work solutions using clientless web-based access?

A) PPTP

B) IPSec IKEv1

C) SSL VPN / HTTPS-based VPN

D) Frame Relay VPN

✔ Correct Answer: C) SSL VPN / HTTPS-based VPN

Explanation: SSL VPNs allow users to securely access corporate resources through a standard web browser using HTTPS, eliminating the need for dedicated VPN client software.

# Section 10: Quality of Service (QoS)

Q81. What is the primary goal of QoS in networking?

A) To increase total bandwidth

B) To prioritize certain types of traffic to meet performance requirements

C) To encrypt all network traffic

D) To prevent unauthorized access

✔ Correct Answer: B) To prioritize certain types of traffic to meet performance requirements

Explanation: QoS mechanisms classify and prioritize traffic types (e.g., voice, video) to ensure latency-sensitive applications receive adequate bandwidth and low delay.

Q82. Which QoS model provides per-flow guarantees using resource reservation?

A) DiffServ

B) Best Effort

C) IntServ

D) MPLS TE

✔ Correct Answer: C) IntServ

Explanation: IntServ (Integrated Services) uses RSVP to reserve network resources along an end-to-end path for specific flows, providing guaranteed QoS but scaling poorly.

Q83. The DSCP (Differentiated Services Code Point) field is located in which IP header field?

A) TTL field

B) Protocol field

C) ToS (Type of Service) byte

D) IP Options field

✔ Correct Answer: C) ToS (Type of Service) byte

Explanation: DSCP uses the 6 most significant bits of the ToS byte in the IPv4 header (or Traffic Class byte in IPv6) to mark packets with traffic class information.

Q84. Which queuing mechanism gives strict priority to delay-sensitive traffic like VoIP?

A) FIFO

B) Weighted Fair Queuing (WFQ)

C) Low Latency Queuing (LLQ)

D) Round Robin

✔ Correct Answer: C) Low Latency Queuing (LLQ)

Explanation: LLQ combines Priority Queuing with Class-Based Weighted Fair Queuing (CBWFQ), giving voice traffic a strict priority queue to minimize delay and jitter.

Q85. What is the difference between traffic shaping and traffic policing?

A) Shaping drops excess traffic; policing queues and delays excess traffic

B) Shaping buffers and delays excess traffic to conform to a rate; policing drops or re-marks excess traffic

C) Both drop excess traffic

D) Policing applies only to inbound traffic

✔ Correct Answer: B) Shaping buffers and delays excess traffic to conform to a rate; policing drops or re-marks excess traffic

Explanation: Traffic shaping smooths bursts by queuing excess packets, causing delay but no drops. Policing enforces a rate by immediately dropping or re-marking packets that exceed the threshold.

Q86. Which metric specifically measures the variation in packet delay over a network path?

A) Latency

B) Throughput

C) Jitter

D) Packet loss

✔ Correct Answer: C) Jitter

Explanation: Jitter is the variation in end-to-end delay between packets in a flow. High jitter degrades real-time applications like VoIP and video conferencing.

Q87. The Expedited Forwarding (EF) DSCP value (101110) is designed for:

A) Best-effort traffic

B) Low-latency, low-jitter, low-loss traffic such as VoIP

C) Network management traffic

D) Bulk file transfers

✔ Correct Answer: B) Low-latency, low-jitter, low-loss traffic such as VoIP

Explanation: EF PHB (Per-Hop Behavior) with DSCP 46 (101110) ensures minimal queuing delay and is the standard QoS marking for real-time voice and video applications.

Q88. What is the purpose of WRED (Weighted Random Early Detection)?

A) To randomly encrypt packets

B) To proactively drop lower-priority packets before queues fill completely to prevent TCP global synchronization

C) To prioritize routing updates

D) To detect wireless interference

✔ Correct Answer: B) To proactively drop lower-priority packets before queues fill completely to prevent TCP global synchronization

Explanation: WRED randomly discards packets before queue overflow, causing individual TCP flows to reduce their rate independently, preventing the TCP global synchronization problem.

# Section 11: Wireless Networks (IEEE 802.11)

Q89. Which IEEE 802.11 standard introduced the 5 GHz frequency band?

A) 802.11b

B) 802.11a

C) 802.11g

D) 802.11n

✔ Correct Answer: B) 802.11a

Explanation: IEEE 802.11a operates on the 5 GHz band and supports up to 54 Mbps. It was ratified simultaneously with 802.11b but uses OFDM rather than DSSS.

Q90. What is the maximum theoretical throughput of IEEE 802.11ac (Wi-Fi 5)?

A) 54 Mbps

B) 600 Mbps

C) Up to ~6.9 Gbps (multi-station)

D) 1 Gbps

✔ Correct Answer: C) Up to ~6.9 Gbps (multi-station)

Explanation: 802.11ac (Wi-Fi 5) can achieve up to ~6.9 Gbps aggregate throughput using multi-user MIMO (MU-MIMO), 160 MHz channels, and 256-QAM on the 5 GHz band.

Q91. CSMA/CA is used in Wi-Fi networks to:

A) Detect and correct bit errors

B) Avoid collisions by checking the medium before transmitting and using random backoff

C) Route packets between wireless cells

D) Authenticate wireless clients

✔ Correct Answer: B) Avoid collisions by checking the medium before transmitting and using random backoff

Explanation: CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance) reduces collisions in wireless networks by having stations listen before transmitting and wait a random backoff time if the medium is busy.

Q92. What is the role of an Access Point (AP) in infrastructure mode?

A) To connect two wired networks

B) To act as a central hub connecting wireless clients to a wired network

C) To assign IP addresses via DHCP

D) To perform NAT for wireless clients

✔ Correct Answer: B) To act as a central hub connecting wireless clients to a wired network

Explanation: In infrastructure mode, an AP bridges wireless clients to the wired LAN, managing medium access, authentication, and associating client devices.

Q93. What does the SSID represent in a wireless network?

A) The encryption key

B) The logical name identifier of a wireless network

C) The channel frequency in MHz

D) The MAC address of the access point

✔ Correct Answer: B) The logical name identifier of a wireless network

Explanation: SSID (Service Set Identifier) is the human-readable name broadcasted by an AP to identify the wireless network. Clients use this name to connect to a specific WLAN.

Q94. Which 802.11 feature allows multiple antennas to improve throughput by sending multiple spatial streams simultaneously?

A) OFDM

B) DSSS

C) MIMO

D) FHSS

✔ Correct Answer: C) MIMO

Explanation: MIMO (Multiple-Input Multiple-Output) uses multiple antennas to transmit and receive several data streams simultaneously, increasing throughput and improving signal reliability.

Q95. What does a hidden node problem cause in Wi-Fi networks?

A) Clients cannot discover the SSID

B) Two clients unaware of each other's transmissions both send simultaneously, causing collisions at the AP

C) The AP cannot forward traffic to the wired network

D) Signal strength drops below threshold

✔ Correct Answer: B) Two clients unaware of each other's transmissions both send simultaneously, causing collisions at the AP

Explanation: The hidden node problem occurs when two stations cannot hear each other but both are in range of the AP; RTS/CTS mechanism is used to mitigate this.

Q96. Which 802.11 standard introduced Wi-Fi 6 (802.11ax) as an improvement over Wi-Fi 5?

A) 802.11ac Wave 2

B) 802.11ax

C) 802.11ad

D) 802.11n

✔ Correct Answer: B) 802.11ax

Explanation: 802.11ax (Wi-Fi 6) improves efficiency in dense environments using OFDMA, BSS Coloring, Target Wake Time (TWT), and MU-MIMO in both 2.4 and 5 GHz bands.

# Section 12: Wireless Security

Q97. WEP was deprecated because:

A) It was too slow for modern networks

B) Its RC4 implementation had critical weaknesses that allow key recovery in minutes

C) It only supported 2.4 GHz

D) It lacked support for AES

✔ Correct Answer: B) Its RC4 implementation had critical weaknesses that allow key recovery in minutes

Explanation: WEP's use of RC4 with static, short keys and weak IV reuse allowed attackers to recover the encryption key by capturing enough packets, making it cryptographically broken.

Q98. WPA2 uses which encryption protocol for data confidentiality?

A) TKIP

B) RC4

C) AES-CCMP

D) DES

✔ Correct Answer: C) AES-CCMP

Explanation: WPA2 uses AES with CCMP (Counter Mode CBC-MAC Protocol) for strong encryption and data integrity, replacing WPA's TKIP and the broken WEP encryption.

Q99. What is an evil twin attack in wireless security?

A) Capturing wireless frames without association

B) Setting up a rogue AP with the same SSID as a legitimate AP to trick clients into connecting

C) Flooding the wireless channel with deauthentication frames

D) Cracking WPA2 using a dictionary attack

✔ Correct Answer: B) Setting up a rogue AP with the same SSID as a legitimate AP to trick clients into connecting

Explanation: An evil twin attack creates a malicious AP that mimics a legitimate one, causing clients to connect to the attacker's AP where traffic can be intercepted.

Q100. IEEE 802.1X provides:

A) Wireless channel bonding

B) Port-based Network Access Control (NAC) using EAP for authentication

C) VLAN tagging over wireless links

D) QoS for wireless traffic

✔ Correct Answer: B) Port-based Network Access Control (NAC) using EAP for authentication

Explanation: IEEE 802.1X provides port-based authentication using EAP (Extensible Authentication Protocol), requiring users/devices to authenticate to a RADIUS server before gaining network access.

Q101. WPA3 introduces which feature to protect against offline dictionary attacks on captured handshakes?

A) CCMP-256

B) SAE (Simultaneous Authentication of Equals)

C) TKIP enhancement

D) WEP compatibility mode

✔ Correct Answer: B) SAE (Simultaneous Authentication of Equals)

Explanation: WPA3 replaces PSK with SAE (based on the Dragonfly handshake), which provides forward secrecy and resistance to offline dictionary attacks even if the handshake is captured.

Q102. A de-authentication attack in wireless networks:

A) Captures WPA2 handshakes by forcing clients to re-authenticate

B) Encrypts wireless traffic with a stolen key

C) Assigns false IP addresses to wireless clients

D) Permanently blocks clients from associating with an AP

✔ Correct Answer: A) Captures WPA2 handshakes by forcing clients to re-authenticate

Explanation: Attackers send spoofed deauthentication frames to disconnect clients from an AP, forcing them to re-associate and reveal the 4-way WPA2 handshake for offline cracking.

Q103. What is the role of a RADIUS server in enterprise wireless authentication?

A) To assign IP addresses to wireless clients

B) To centrally authenticate users and authorize network access using 802.1X/EAP

C) To broadcast the SSID

D) To perform wireless site surveys

✔ Correct Answer: B) To centrally authenticate users and authorize network access using 802.1X/EAP

Explanation: In enterprise Wi-Fi, the AP forwards client credentials to a RADIUS server which validates them against a directory (e.g., Active Directory), returning access/deny decisions to the AP.

Q104. Which wireless attack passively monitors wireless traffic without transmitting any frames?

A) Deauthentication attack

B) Evil twin

C) Passive eavesdropping / packet sniffing

D) ARP poisoning

✔ Correct Answer: C) Passive eavesdropping / packet sniffing

Explanation: Passive eavesdropping places a wireless adapter in monitor mode to capture all 802.11 frames in range without transmitting, making it undetectable to standard intrusion detection.

# Section 13: Network Devices

Q105. What is the key difference between a hub and a switch?

A) A hub uses IP addresses; a switch uses MAC addresses

B) A hub broadcasts all frames to all ports; a switch intelligently forwards frames based on MAC addresses

C) A hub operates at Layer 3; a switch at Layer 2

D) A hub supports VLANs; a switch does not

✔ Correct Answer: B) A hub broadcasts all frames to all ports; a switch intelligently forwards frames based on MAC addresses

Explanation: Hubs are Layer 1 devices that repeat signals to all ports (creating a single collision domain). Switches are Layer 2 devices that maintain MAC tables and forward frames only to the correct port.

Q106. A router's primary function is to:

A) Filter traffic based on MAC addresses

B) Forward packets between different IP networks using routing tables

C) Provide wireless connectivity to end devices

D) Amplify network signals over long distances

✔ Correct Answer: B) Forward packets between different IP networks using routing tables

Explanation: Routers operate at Layer 3, examining destination IP addresses and using routing tables to determine the best path to forward packets between different networks.

Q107. A stateful firewall tracks:

A) Only source IP addresses

B) The state of active network connections to allow return traffic

C) Only TCP flags in headers

D) DNS queries from internal hosts

✔ Correct Answer: B) The state of active network connections to allow return traffic

Explanation: A stateful firewall maintains a state table tracking active connections. It allows established session return traffic automatically, unlike packet filters which inspect each packet independently.

Q108. What does a load balancer do?

A) Balances VLAN assignments across switches

B) Distributes incoming network traffic across multiple servers to improve availability and performance

C) Provides redundant routing between sites

D) Encrypts traffic between data centers

✔ Correct Answer: B) Distributes incoming network traffic across multiple servers to improve availability and performance

Explanation: Load balancers distribute client requests across a server pool using algorithms like round-robin, least connections, or IP hash to prevent any single server from becoming a bottleneck.

Q109. An IDS (Intrusion Detection System) differs from an IPS (Intrusion Prevention System) in that:

A) IDS blocks threats; IPS only alerts

B) IDS monitors and alerts; IPS monitors, alerts, and can actively block threats

C) IPS uses signatures; IDS uses anomaly detection only

D) They are functionally identical

✔ Correct Answer: B) IDS monitors and alerts; IPS monitors, alerts, and can actively block threats

Explanation: An IDS passively monitors traffic and generates alerts for suspicious activity. An IPS is deployed inline and can actively block or modify malicious traffic in real time.

Q110. Which device connects multiple networks together and operates at all seven OSI layers?

A) Switch

B) Router

C) Gateway

D) Hub

✔ Correct Answer: C) Gateway

Explanation: A gateway can operate at any or all OSI layers and translates between different network architectures or protocols, acting as an entry/exit point between dissimilar systems.

Q111. A proxy server primarily serves to:

A) Encrypt all outgoing traffic

B) Act as an intermediary between clients and servers, providing caching, filtering, and anonymity

C) Assign IP addresses to hosts

D) Perform Layer 3 routing

✔ Correct Answer: B) Act as an intermediary between clients and servers, providing caching, filtering, and anonymity

Explanation: Proxy servers intercept client requests, forwarding them to destination servers on behalf of clients. They can cache content, filter URLs, and hide client IP addresses.

Q112. What is the function of a wireless LAN controller (WLC)?

A) To provide DHCP addresses to wireless clients

B) To centrally manage and configure multiple access points in an enterprise wireless network

C) To perform WPA3 encryption

D) To route traffic between wireless VLANs

✔ Correct Answer: B) To centrally manage and configure multiple access points in an enterprise wireless network

Explanation: A WLC centralizes the management of APs (configurations, firmware, RF settings, security policies) and handles client roaming, allowing lightweight APs to offload control functions.

Q113. What does a content delivery network (CDN) node function as in terms of network devices?

A) A centralized storage server

B) A geographically distributed cache/proxy server that reduces latency by serving content closer to users

C) A dedicated VPN endpoint

D) A DNS root server

✔ Correct Answer: B) A geographically distributed cache/proxy server that reduces latency by serving content closer to users

Explanation: CDN nodes cache and serve content from locations geographically closer to end users, reducing round-trip times, offloading origin servers, and improving user experience.

Q114. A Layer 3 switch differs from a traditional router in that:

A) It cannot perform routing

B) It performs routing in hardware (ASICs) for high-speed inter-VLAN routing alongside switching

C) It only supports static routes

D) It operates exclusively at Layer 2

✔ Correct Answer: B) It performs routing in hardware (ASICs) for high-speed inter-VLAN routing alongside switching

Explanation: Layer 3 switches use dedicated hardware (ASICs) to perform IP routing at wire speed, combining switch port density with routing capability for efficient inter-VLAN routing.

# Section 14: TCP & UDP

Q115. Which TCP flag is used to initiate a connection?

A) ACK

B) FIN

C) SYN

D) RST

✔ Correct Answer: C) SYN

Explanation: The SYN (synchronize) flag initiates a TCP connection. The three-way handshake begins with SYN, is acknowledged with SYN-ACK, and completed with ACK.

Q116. TCP guarantees delivery through:

A) Checksum verification only

B) Sequence numbers, acknowledgments, and retransmission of lost segments

C) Increasing packet TTL

D) Using broadcast acknowledgments

✔ Correct Answer: B) Sequence numbers, acknowledgments, and retransmission of lost segments

Explanation: TCP uses sequence numbers to order data and acknowledgments to confirm receipt. If a segment is unacknowledged within a timeout, TCP retransmits it.

Q117. UDP is preferred over TCP for real-time applications because:

A) UDP provides better error correction

B) UDP has lower overhead and latency since it lacks connection establishment and retransmission

C) UDP supports larger packet sizes

D) UDP uses encryption by default

✔ Correct Answer: B) UDP has lower overhead and latency since it lacks connection establishment and retransmission

Explanation: UDP is connectionless and stateless with minimal overhead. For real-time apps (VoIP, video streaming), a slight packet loss is acceptable but added delay from TCP retransmission is not.

Q118. What is the TCP sliding window mechanism used for?

A) To encrypt TCP segments

B) To control the flow of data by allowing multiple unacknowledged segments in transit

C) To select the best network route

D) To segment data into equal-sized packets

✔ Correct Answer: B) To control the flow of data by allowing multiple unacknowledged segments in transit

Explanation: The sliding window allows a sender to transmit multiple segments before waiting for acknowledgment, improving throughput by keeping the network pipe full.

Q119. A TCP connection is closed using which process?

A) Two-way handshake with RST

B) Four-way handshake with FIN and ACK exchanges

C) One-way FIN from the server

D) SYN timeout

✔ Correct Answer: B) Four-way handshake with FIN and ACK exchanges

Explanation: TCP uses a four-way termination: FIN from one side, ACK from the other, FIN from the second side, then a final ACK—allowing both sides to close gracefully.

Q120. What is the maximum size of a UDP datagram payload?

A) 512 bytes

B) 65,507 bytes

C) 1,500 bytes

D) Unlimited

✔ Correct Answer: B) 65,507 bytes

Explanation: UDP header is 8 bytes. IPv4 max payload is 65,535 bytes. So max UDP payload = 65,535 – 8 (UDP header) – 20 (IP header) = 65,507 bytes.

Q121. Which protocol uses both TCP and UDP on port 53?

A) HTTP

B) SNMP

C) DNS

D) FTP

✔ Correct Answer: C) DNS

Explanation: DNS uses UDP port 53 for most queries (fast, low overhead) and switches to TCP port 53 for responses exceeding 512 bytes or for zone transfers.

Q122. What does the TCP RST flag indicate?

A) Requesting retransmission of a lost segment

B) Immediately aborting a connection

C) Requesting a connection be established

D) Indicating the final segment of data

✔ Correct Answer: B) Immediately aborting a connection

Explanation: RST (Reset) abruptly terminates a TCP connection without waiting for pending data, used when an error occurs or when a connection request arrives for a closed port.

Q123. Which TCP option allows both sides to negotiate a larger window size beyond the 16-bit limit?

A) Selective ACK (SACK)

B) Window Scale option

C) Timestamps option

D) Maximum Segment Size (MSS)

✔ Correct Answer: B) Window Scale option

Explanation: The TCP Window Scale option (RFC 7323) extends the 16-bit window field by a scale factor, enabling window sizes up to 1 GB for high-bandwidth, high-latency links.

Q124. What is the purpose of Nagle's algorithm in TCP?

A) To prevent SYN flood attacks

B) To reduce the number of small packets by buffering data until an acknowledgment or enough data accumulates

C) To detect duplicate ACKs

D) To calculate the retransmission timeout

✔ Correct Answer: B) To reduce the number of small packets by buffering data until an acknowledgment or enough data accumulates

Explanation: Nagle's algorithm improves network efficiency by coalescing small writes into a single segment, reducing the overhead of many small packets (tinygrams) on the network.

# Section 15: Congestion Control

Q125. TCP congestion control begins with which phase after connection establishment?

A) Congestion Avoidance

B) Fast Recovery

C) Slow Start

D) Fast Retransmit

✔ Correct Answer: C) Slow Start

Explanation: Slow Start begins with a Congestion Window (cwnd) of 1 MSS and doubles it each RTT until the slow-start threshold (ssthresh) is reached or congestion is detected.

Q126. In TCP, what triggers the Fast Retransmit mechanism?

A) A retransmission timeout

B) Receipt of three duplicate ACKs

C) Window size reaching zero

D) TTL expiration

✔ Correct Answer: B) Receipt of three duplicate ACKs

Explanation: When a sender receives three duplicate ACKs, it assumes the segment was lost and retransmits it immediately without waiting for the retransmission timer to expire.

Q127. What does TCP Cubic optimize compared to TCP Reno?

A) It uses UDP for faster recovery

B) It uses a cubic function for window growth, optimized for high-bandwidth delay-product networks

C) It adds encryption to TCP headers

D) It eliminates the slow-start phase entirely

✔ Correct Answer: B) It uses a cubic function for window growth, optimized for high-bandwidth delay-product networks

Explanation: TCP Cubic uses a cubic window growth function that is independent of RTT, allowing faster window increase on high-BDP links while being fair to other flows.

Q128. Explicit Congestion Notification (ECN) allows routers to:

A) Drop packets silently when congested

B) Signal congestion to endpoints via header bits without dropping packets

C) Increase the queue size dynamically

D) Bypass TCP congestion control

✔ Correct Answer: B) Signal congestion to endpoints via packet header bits without dropping packets

Explanation: ECN uses two bits in the IP ToS and TCP headers to notify endpoints of congestion, allowing them to reduce their sending rate without the packet loss that triggers normal congestion control.

Q129. What happens to TCP's congestion window (cwnd) when a retransmission timeout occurs in TCP Reno?

A) cwnd is halved

B) cwnd is reduced by 1 MSS

C) cwnd is reset to 1 MSS and ssthresh is set to half the previous cwnd

D) cwnd remains unchanged

✔ Correct Answer: C) cwnd is reset to 1 MSS and ssthresh is set to half the previous cwnd

Explanation: A timeout is treated as a severe congestion signal; TCP Reno resets cwnd to 1 MSS, sets ssthresh to half the cwnd at the time of timeout, and restarts slow start.

Q130. What is the TCP Global Synchronization problem?

A) All TCP connections using the same NTP server fail simultaneously

B) Many TCP flows simultaneously reduce their window size after tail-drop, causing underutilization followed by synchronized bursts

C) TCP fails to synchronize sequence numbers during slow start

D) Multiple ACKs arrive out of order

✔ Correct Answer: B) Many TCP flows simultaneously reduce their window size after tail-drop, causing underutilization followed by synchronized bursts

Explanation: When a router's queue fills and uses tail-drop, many TCP connections simultaneously detect loss and back off, causing bandwidth underutilization followed by synchronized bursts—addressed by RED/WRED.

Q131. Which TCP variant is designed for data center networks with very low latency requirements?

A) TCP Reno

B) TCP BBR

C) TCP Tahoe

D) TCP Vegas

✔ Correct Answer: B) TCP BBR

Explanation: TCP BBR (Bottleneck Bandwidth and Round-trip propagation time) by Google models the network to estimate bandwidth and RTT directly, achieving high throughput at low latency without relying on packet loss as a congestion signal.

Q132. During TCP Congestion Avoidance, cwnd grows by approximately:

A) 1 MSS per ACK

B) 2 MSS per RTT

C) 1 MSS per RTT (additive increase)

D) It doubles each RTT

✔ Correct Answer: C) 1 MSS per RTT (additive increase)

Explanation: In Congestion Avoidance (AIMD), cwnd increases by 1 MSS per RTT (additive increase) to gently probe for additional bandwidth, and is halved on detecting congestion (multiplicative decrease).

# Section 16: Packet Switching & Routing Algorithms

Q133. In packet switching, data is divided into packets that:

A) All follow the same predetermined path

B) Can take different routes through the network and are reassembled at the destination

C) Are sent as a continuous bit stream

D) Reserve bandwidth along the entire path before transmission

✔ Correct Answer: B) Can take different routes through the network and are reassembled at the destination

Explanation: Packet switching routes each packet independently through the network. Packets may arrive out of order and are reassembled by the receiving application, making efficient use of shared links.

Q134. What is the main advantage of packet switching over circuit switching?

A) Guaranteed bandwidth and delay

B) Statistical multiplexing allows efficient sharing of bandwidth across multiple users

C) No packet loss occurs

D) Simpler network equipment required

✔ Correct Answer: B) Statistical multiplexing allows efficient sharing of bandwidth across multiple users

Explanation: Packet switching uses statistical multiplexing—bandwidth is shared dynamically, so multiple users can use the full link when others are idle, improving overall utilization compared to dedicated circuits.

Q135. Dijkstra's algorithm is used by which routing protocol?

A) RIP

B) OSPF

C) BGP

D) EIGRP

✔ Correct Answer: B) OSPF

Explanation: OSPF uses Dijkstra's Shortest Path First (SPF) algorithm to compute the shortest path tree from the local router to all destinations based on the link-state database.

Q136. What is store-and-forward switching?

A) Forwarding packets before they are fully received

B) Receiving the entire frame, checking for errors, then forwarding it

C) Storing all frames until midnight before forwarding

D) A method used only by hubs

✔ Correct Answer: B) Receiving the entire frame, checking for errors, then forwarding it

Explanation: Store-and-forward switches receive the complete frame, verify its integrity using CRC, then forward it. This adds latency but eliminates propagation of corrupted frames.

Q137. Which switching method has the lowest latency but may forward corrupted frames?

A) Store-and-forward

B) Fragment-free

C) Cut-through

D) Store-and-discard

✔ Correct Answer: C) Cut-through

Explanation: Cut-through switching begins forwarding a frame as soon as it reads the destination MAC address (after 6 bytes), providing the lowest latency but potentially forwarding corrupted frames.

Q138. What is a routing loop and how does Poison Reverse help prevent it?

A) A routing loop is a misconfigured static route; Poison Reverse blocks the misconfigured route

B) A routing loop is when packets cycle indefinitely; Poison Reverse advertises the route with infinite metric back through the interface it was learned from

C) A routing loop causes TTL to increase; Poison Reverse resets TTL

D) Poison Reverse is used only in OSPF

✔ Correct Answer: B) A routing loop is when packets cycle indefinitely; Poison Reverse advertises the route with infinite metric back through the interface it was learned from

Explanation: Poison Reverse improves on split horizon by explicitly advertising a route with metric 16 (infinity for RIP) back through the interface it was learned from, more aggressively preventing loops.

Q139. MPLS (Multiprotocol Label Switching) improves routing by:

A) Using IP addresses for all forwarding decisions

B) Inserting a fixed-length label between Layer 2 and Layer 3 headers for fast hardware-based forwarding

C) Encrypting all traffic between provider routers

D) Replacing BGP in service provider networks

✔ Correct Answer: B) Inserting a fixed-length label between Layer 2 and Layer 3 headers for fast hardware-based forwarding

Explanation: MPLS assigns labels to packets at the network edge; interior routers (LSRs) swap labels for forwarding decisions, enabling fast hardware switching, traffic engineering, and VPN services.

Q140. What does a TTL (Time to Live) field in an IP packet prevent?

A) Packet fragmentation

B) Routing loops from consuming network resources indefinitely

C) Out-of-order packet delivery

D) IP address spoofing

✔ Correct Answer: B) Routing loops from consuming network resources indefinitely

Explanation: TTL is decremented by each router. When it reaches zero, the router discards the packet and sends an ICMP Time Exceeded message, preventing packets from looping forever.

Q141. What is the Bellman-Ford algorithm's time complexity for a graph with V vertices and E edges?

A) O(V log V)

B) O(V²)

C) O(VE)

D) O(E log E)

✔ Correct Answer: C) O(VE)

Explanation: Bellman-Ford has a time complexity of O(VE) where V is vertices and E is edges. It converges in at most V-1 iterations, checking all E edges each iteration.

Q142. In IP networks, what is a 'black hole' route?

A) A route that encrypts all traffic

B) A route that silently discards packets directed to it without sending ICMP unreachable messages

C) A route only used for multicast traffic

D) A backup route activated by failover

✔ Correct Answer: B) A route that silently discards packets directed to it without sending ICMP unreachable messages

Explanation: A null/black hole route sends packets to a discard interface (Null0 in Cisco), dropping them silently. Used to block traffic without generating ICMP, often to prevent DoS amplification.

# Section 17: IPv6

Q143. How many bits long is an IPv6 address?

A) 32 bits

B) 64 bits

C) 128 bits

D) 256 bits

✔ Correct Answer: C) 128 bits

Explanation: IPv6 addresses are 128 bits long, written as eight groups of four hexadecimal digits, providing 2^128 (~3.4×10^38) unique addresses.

Q144. What is the IPv6 equivalent of the IPv4 broadcast address?

A) FF02::1

B) ::1

C) FE80::1

D) IPv6 has no broadcast

✔ Correct Answer: D) IPv6 has no broadcast

Explanation: IPv6 eliminates broadcast; instead, it uses multicast (FF02::1 for all nodes) and anycast addresses. This reduces unnecessary traffic processing by all network hosts.

Q145. Which IPv6 address type is used for link-local communication?

A) 2001::/16 (Global Unicast)

B) FC00::/7 (Unique Local)

C) FE80::/10 (Link-Local)

D) FF02::/16 (Multicast)

✔ Correct Answer: C) FE80::/10 (Link-Local)

Explanation: Link-local addresses (FE80::/10) are automatically configured on all IPv6 interfaces and are valid only within a single network segment; they are not routable beyond the local link.

Q146. SLAAC (Stateless Address Autoconfiguration) allows an IPv6 host to:

A) Obtain an IPv6 address from a DHCPv6 server

B) Automatically generate a global unicast address using the network prefix and its EUI-64 interface ID

C) Discover DNS servers without any protocol

D) Convert IPv4 addresses to IPv6

✔ Correct Answer: B) Automatically generate a global unicast address using the network prefix and its EUI-64 interface ID

Explanation: SLAAC enables hosts to autonomously configure IPv6 addresses by combining the /64 prefix from Router Advertisements with a self-generated 64-bit interface identifier.

Q147. Which IPv6 mechanism replaces ARP for address resolution?

A) RARP

B) DHCPv6

C) Neighbor Discovery Protocol (NDP)

D) ICMP Echo

✔ Correct Answer: C) Neighbor Discovery Protocol (NDP)

Explanation: NDP (defined in RFC 4861) uses ICMPv6 messages (Neighbor Solicitation/Advertisement) to perform address resolution, replacing IPv4's ARP while adding router discovery and SLAAC.

Q148. The compressed IPv6 address 2001:0DB8:0000:0000:0000:0000:0000:0001 can be written as:

A) 2001:DB8::1

B) 2001:DB8:0:1

C) 2001::DB8::1

D) 2001:DB8:0000::0001

✔ Correct Answer: A) 2001:DB8::1

Explanation: IPv6 compression rules: leading zeros in each group are dropped (0DB8→DB8), and consecutive all-zero groups are replaced with :: (only once). Result: 2001:DB8::1.

Q149. What is the purpose of IPv6 Unique Local Addresses (ULA)?

A) For Internet-routable global communication

B) For private internal communication, similar to RFC 1918 IPv4 private addresses

C) For link-local neighbor discovery only

D) Reserved for multicast groups

✔ Correct Answer: B) For private internal communication, similar to RFC 1918 IPv4 private addresses

Explanation: ULA addresses (FC00::/7, commonly FD00::/8) are for internal organization use and not routable on the global Internet, serving the same purpose as IPv4's private address space.

Q150. Dual-stack is an IPv6 transition mechanism where:

A) An IPv4 packet is tunneled inside an IPv6 packet

B) Devices run both IPv4 and IPv6 simultaneously on the same interfaces

C) IPv6 is translated to IPv4 at the network edge

D) Only DNS64 is used to resolve addresses

✔ Correct Answer: B) Devices run both IPv4 and IPv6 simultaneously on the same interfaces

Explanation: Dual-stack allows devices to operate both IPv4 and IPv6 protocols concurrently, enabling gradual migration to IPv6 while maintaining full IPv4 compatibility.

# Section 18: WAN Technologies

Q151. What is the primary characteristic of a leased line WAN connection?

A) Bandwidth shared with other customers

B) A dedicated, point-to-point circuit with guaranteed bandwidth between two sites

C) Uses best-effort delivery like the Internet

D) Only available for wireless connections

✔ Correct Answer: B) A dedicated, point-to-point circuit with guaranteed bandwidth between two sites

Explanation: Leased lines provide a permanent, dedicated, full-duplex connection between two locations with fixed bandwidth, ensuring consistent performance but at higher cost.

Q152. MPLS VPNs are preferred in enterprise WANs because:

A) They are free to use over the public Internet

B) They provide QoS guarantees, traffic engineering, and secure segmentation over carrier networks

C) They replace all need for routing protocols

D) They only work with IPv6

✔ Correct Answer: B) They provide QoS guarantees, traffic engineering, and secure segmentation over carrier networks

Explanation: MPLS VPNs in service provider networks deliver QoS, predictable latency, and logical traffic separation between customers—advantages over Internet-based VPNs.

Q153. SD-WAN (Software-Defined WAN) primarily enables:

A) Only hardware-based WAN acceleration

B) Centralized policy-based management of multiple WAN links using software

C) Direct fiber connections between branch offices

D) Automatic conversion of MPLS to broadband

✔ Correct Answer: B) Centralized policy-based management of multiple WAN links using software

Explanation: SD-WAN decouples WAN control from hardware, enabling centralized management of multiple transport links (MPLS, broadband, LTE) with intelligent traffic steering based on application policies.

Q154. Frame Relay is a WAN technology that operates at which OSI layer?

A) Layer 1

B) Layer 2

C) Layer 3

D) Layer 4

✔ Correct Answer: B) Layer 2

Explanation: Frame Relay is a Layer 2 (Data Link) packet-switched WAN protocol that uses virtual circuits identified by DLCIs (Data Link Connection Identifiers) for multiplexing traffic.

Q155. What does DSL (Digital Subscriber Line) use as its transmission medium?

A) Coaxial cable (same as cable TV)

B) Fiber optic cable exclusively

C) Existing copper telephone lines (PSTN)

D) Satellite links

✔ Correct Answer: C) Existing copper telephone lines (PSTN)

Explanation: DSL transmits digital data over existing copper telephone lines at high frequency, allowing simultaneous voice and data transmission on the same physical pair.

Q156. Which WAN technology uses a geostationary satellite and has high latency (~600ms RTT) as a drawback?

A) Fiber WAN

B) MPLS

C) Traditional satellite broadband

D) Metro Ethernet

✔ Correct Answer: C) Traditional satellite broadband

Explanation: Geostationary satellites orbit at ~35,786 km altitude, causing significant propagation delay (~270ms one-way, ~600ms RTT), making them unsuitable for latency-sensitive applications.

Q157. What is the purpose of a CSU/DSU in WAN connectivity?

A) To encrypt WAN traffic

B) To interface customer equipment (DTE) with the carrier's digital WAN circuit (DCE)

C) To perform NAT for WAN traffic

D) To provide wireless WAN access

✔ Correct Answer: B) To interface customer equipment (DTE) with the carrier's digital WAN circuit (DCE)

Explanation: A CSU/DSU (Channel Service Unit/Data Service Unit) terminates the digital circuit from the carrier at the customer premises, converting signal formats between the LAN equipment and WAN circuit.

Q158. Metro Ethernet provides WAN connectivity using:

A) Frame Relay virtual circuits

B) Ethernet technology extended over a metropolitan-area carrier network

C) ATM cells on fiber

D) SONET multiplexing

✔ Correct Answer: B) Ethernet technology extended over a metropolitan-area carrier network

Explanation: Metro Ethernet extends standard Ethernet (IEEE 802.3) over carrier networks within a metropolitan area, offering familiar interfaces, scalable bandwidth, and lower cost than traditional WAN technologies.

# Section 19: Cloud Networking Basics

Q159. A VPC (Virtual Private Cloud) in cloud networking is:

A) A physical server dedicated to one customer

B) A logically isolated virtual network within a public cloud provider's infrastructure

C) A VPN between two cloud providers

D) A cloud-based load balancer

✔ Correct Answer: B) A logically isolated virtual network within a public cloud provider's infrastructure

Explanation: A VPC provides a logically isolated section of the cloud where users can launch resources, define IP ranges, create subnets, configure route tables, and control access.

Q160. What is an internet gateway in a cloud VPC used for?

A) To connect the VPC to on-premises networks via VPN

B) To enable communication between VPC resources and the public Internet

C) To route traffic between VPC subnets

D) To provide DHCP to VPC instances

✔ Correct Answer: B) To enable communication between VPC resources and the public Internet

Explanation: An Internet Gateway is a horizontally scaled, highly available VPC component that allows communication between VPC instances and the Internet, performing NAT for instances with public IPs.

Q161. Which cloud networking model describes a dedicated private connection from an enterprise's premises to a cloud provider?

A) Public Internet transit

B) Direct Connect / ExpressRoute

C) VPN over Internet

D) CDN peering

✔ Correct Answer: B) Direct Connect / ExpressRoute

Explanation: Services like AWS Direct Connect and Azure ExpressRoute provide dedicated private network connections (bypassing the public Internet) between enterprise data centers and cloud providers, offering lower latency and predictable bandwidth.

Q162. In cloud networking, a security group functions primarily as:

A) A physical firewall appliance

B) A stateful virtual firewall controlling inbound/outbound traffic for cloud instances

C) A routing table for cloud subnets

D) A load balancer for cloud applications

✔ Correct Answer: B) A stateful virtual firewall controlling inbound/outbound traffic for cloud instances

Explanation: Cloud security groups act as virtual firewalls at the instance level, evaluating rules statefully (allowing return traffic automatically) for each network interface.

Q163. What is VPC peering?

A) Connecting a VPC to the Internet

B) A networking connection between two VPCs that enables private routing between them

C) A load balancing arrangement between cloud regions

D) An encrypted tunnel using IPSec between cloud accounts

✔ Correct Answer: B) A networking connection between two VPCs that enables private routing between them

Explanation: VPC peering creates a private networking connection between two VPCs (same or different accounts/regions) allowing resources to communicate using private IP addresses as if on the same network.

Q164. Which model best describes a cloud provider offering virtualized network infrastructure (switches, routers) as a service?

A) SaaS

B) PaaS

C) IaaS

D) NaaS

✔ Correct Answer: D) NaaS

Explanation: NaaS (Network as a Service) provides network infrastructure and services (routing, switching, firewalling) on-demand through a cloud model, often without the customer managing physical hardware.

Q165. An Availability Zone (AZ) in cloud networking provides:

A) Geographic load balancing across continents

B) Isolated physical infrastructure within a region for fault tolerance

C) Content delivery via distributed caches

D) Dedicated IPv6 subnets

✔ Correct Answer: B) Isolated physical infrastructure within a region for fault tolerance

Explanation: Availability Zones are physically isolated data centers within a cloud region, each with independent power, cooling, and networking. Deploying across AZs ensures application resilience to single-point failures.

Q166. A cloud Transit Gateway simplifies networking by:

A) Providing direct Internet access to all subnets

B) Acting as a hub to interconnect multiple VPCs and on-premises networks through a single connection point

C) Encrypting all traffic between VPCs

D) Replacing all routing protocols in the cloud

✔ Correct Answer: B) Acting as a hub to interconnect multiple VPCs and on-premises networks through a single connection point

Explanation: A Transit Gateway acts as a regional network hub, allowing transitive peering between thousands of VPCs and on-premises networks without complex mesh VPC peering arrangements.

# Section 20: Network Security Basics

Q167. What is a DDoS (Distributed Denial of Service) attack?

A) Encrypting victim's data and demanding ransom

B) Using multiple compromised systems to overwhelm a target with traffic, making it unavailable

C) Intercepting communications between two parties

D) Exploiting a zero-day vulnerability in server software

✔ Correct Answer: B) Using multiple compromised systems to overwhelm a target with traffic, making it unavailable

Explanation: DDoS attacks use botnets (many compromised machines) to flood a target with traffic or requests, exhausting its resources and preventing legitimate users from accessing the service.

Q168. What is the principle of least privilege in network security?

A) All users have administrator access

B) Users and systems are granted only the minimum access rights needed to perform their function

C) Passwords must be changed every day

D) All traffic is encrypted by default

✔ Correct Answer: B) Users and systems are granted only the minimum access rights needed to perform their function

Explanation: Least privilege limits potential damage from accidents, errors, or attacks by ensuring accounts and processes operate with only the permissions required for their specific tasks.

Q169. A man-in-the-middle (MitM) attack occurs when:

A) An attacker floods a server with SYN packets

B) An attacker secretly intercepts and potentially alters communication between two parties

C) An attacker guesses passwords through brute force

D) An attacker exploits a buffer overflow vulnerability

✔ Correct Answer: B) An attacker secretly intercepts and potentially alters communication between two parties

Explanation: In a MitM attack, the attacker positions themselves between two communicating hosts, able to read, modify, or inject data into the communication stream without either party's knowledge.

Q170. Which security protocol provides network-layer authentication and encryption for all IP traffic?

A) SSL/TLS

B) IPSec

C) SSH

D) S/MIME

✔ Correct Answer: B) IPSec

Explanation: IPSec operates at the network layer and provides authentication (AH) and encryption (ESP) for all IP traffic between hosts or networks, transparent to applications.

Q171. What does a honeypot serve in network security?

A) To provide redundant DNS resolution

B) A decoy system designed to lure attackers and study their techniques

C) To distribute load across multiple servers

D) To cache frequently accessed web content

✔ Correct Answer: B) A decoy system designed to lure attackers and study their techniques

Explanation: Honeypots are intentionally vulnerable systems or services set up to attract attackers. By monitoring interactions with honeypots, security teams learn about attack techniques and detect intrusions early.

Q172. An ACL (Access Control List) on a router is used to:

A) Assign IP addresses to interfaces

B) Filter traffic based on defined permit/deny rules for source/destination IP, ports, or protocols

C) Advertise routes to neighbors

D) Authenticate remote administrators

✔ Correct Answer: B) Filter traffic based on defined permit/deny rules for source/destination IP, ports, or protocols

Explanation: ACLs are sequential rule sets applied to router interfaces that inspect packets and either permit or deny them based on criteria like IP addresses, protocols, and port numbers.

Q173. What is port scanning and why is it considered a security concern?

A) Monitoring network bandwidth on each switch port

B) Sending probes to discover open ports on a host, revealing running services that could be targeted

C) Counting packets per switch port for capacity planning

D) Assigning VLAN IDs to switch ports

✔ Correct Answer: B) Sending probes to discover open ports on a host, revealing running services that could be targeted

Explanation: Port scanning identifies which TCP/UDP ports are open on a target system, revealing what services are running. Attackers use this as reconnaissance to find exploitable services.

Q174. Zero Trust networking is based on the principle:

A) Trust all internal network traffic; verify external traffic only

B) Never trust, always verify — no implicit trust based on network location

C) All users must use VPN regardless of location

D) Only devices with valid SSL certificates are trusted

✔ Correct Answer: B) Never trust, always verify — no implicit trust based on network location

Explanation: Zero Trust assumes no user or device is inherently trusted, even inside the network perimeter. Every access request must be authenticated, authorized, and continuously validated.

# Section 21: Common Protocols (HTTP/S, FTP, SSH, SMTP, SNMP, NTP)

Q175. Which HTTP method is used to submit data to a server, typically causing a change in state?

A) GET

B) POST

C) HEAD

D) OPTIONS

✔ Correct Answer: B) POST

Explanation: HTTP POST sends data in the request body to a server, typically to create or update a resource. GET retrieves data without modifying server state.

Q176. HTTPS encrypts HTTP traffic using:

A) SSH

B) IPSec

C) TLS (Transport Layer Security)

D) Kerberos

✔ Correct Answer: C) TLS (Transport Layer Security)

Explanation: HTTPS is HTTP over TLS. TLS encrypts the HTTP traffic, provides server authentication via certificates, and ensures data integrity, protecting against eavesdropping and MitM attacks.

Q177. FTP (File Transfer Protocol) uses which ports?

A) TCP 20 and 21

B) UDP 20 and 21

C) TCP 22 and 23

D) TCP 80 and 443

✔ Correct Answer: A) TCP 20 and 21

Explanation: FTP uses TCP port 21 for control (commands) and TCP port 20 for active-mode data transfer. It transmits data in cleartext and should be replaced by SFTP or FTPS.

Q178. SSH (Secure Shell) operates on which port and provides what security?

A) Port 23, provides cleartext remote access

B) Port 22, provides encrypted remote login and secure file transfer

C) Port 443, provides encrypted web access

D) Port 21, provides secure FTP

✔ Correct Answer: B) Port 22, provides encrypted remote login and secure file transfer

Explanation: SSH uses TCP port 22 and provides encrypted, authenticated remote shell access (replacing Telnet's insecure cleartext), as well as secure file transfer (SFTP/SCP) and port forwarding.

Q179. SMTP (Simple Mail Transfer Protocol) is used for:

A) Retrieving email from a mail server

B) Sending email from a client to a server or between mail servers

C) Encrypting email content

D) Authenticating email users via LDAP

✔ Correct Answer: B) Sending email from a client to a server or between mail servers

Explanation: SMTP (TCP port 25, 587 for submission) handles the sending and relaying of email messages. POP3/IMAP are used for retrieving messages from a mailbox.

Q180. SNMP (Simple Network Management Protocol) uses which transport and ports?

A) TCP 161/162

B) UDP 161 (agent) and UDP 162 (trap receiver)

C) UDP 53

D) TCP 443

✔ Correct Answer: B) UDP 161 (agent) and UDP 162 (trap receiver)

Explanation: SNMP uses UDP port 161 for manager-to-agent queries/responses and UDP port 162 for unsolicited trap notifications from agents to the network management system.

Q181. NTP (Network Time Protocol) is critical in networking because:

A) It provides IP addresses to network devices

B) Accurate time synchronization is essential for logs, security certificates, and authentication protocols like Kerberos

C) It encrypts all time-sensitive network data

D) It manages device firmware updates

✔ Correct Answer: B) Accurate time synchronization is essential for logs, security certificates, and authentication protocols like Kerberos

Explanation: NTP synchronizes device clocks to a reference time source. Clock skew can break SSL/TLS certificates, invalidate Kerberos tickets, and cause log inconsistencies across systems.

Q182. Which version of SNMP introduced strong authentication and encryption?

A) SNMPv1

B) SNMPv2c

C) SNMPv3

D) SNMPv4

✔ Correct Answer: C) SNMPv3

Explanation: SNMPv3 introduced authentication (using HMAC-MD5 or HMAC-SHA) and privacy/encryption (using DES or AES) through its security model, addressing the cleartext community-string weaknesses in v1 and v2c.

Q183. Which protocol is used to securely transfer files and is an extension of SSH?

A) FTPS

B) TFTP

C) SFTP

D) SCP only

✔ Correct Answer: C) SFTP

Explanation: SFTP (SSH File Transfer Protocol) runs over SSH (port 22) providing encrypted file transfer, directory listing, and file management—unlike FTP or TFTP which transmit in cleartext.

Q184. HTTPS uses which default port?

A) 80

B) 8080

C) 443

D) 8443

✔ Correct Answer: C) 443

Explanation: HTTPS uses TCP port 443 by default. HTTP uses TCP port 80. When a browser connects to https://, it establishes a TLS session on port 443 before exchanging HTTP data.

# Section 22: Network Performance Metrics

Q185. What does bandwidth refer to in networking?

A) The time it takes for a packet to travel from source to destination

B) The maximum data transfer rate of a network link

C) The number of packets lost in transit

D) The variation in packet arrival times

✔ Correct Answer: B) The maximum data transfer rate of a network link

Explanation: Bandwidth is the theoretical maximum amount of data that can be transmitted over a network link per unit time (e.g., 1 Gbps), often confused with throughput (actual achieved rate).

Q186. Round-Trip Time (RTT) measures:

A) Time for a packet to travel one way

B) Time for a signal to travel from source to destination and back

C) Time to establish a TCP connection

D) Packet queuing delay only

✔ Correct Answer: B) Time for a signal to travel from source to destination and back

Explanation: RTT is the total time for a packet to travel from sender to receiver and for an acknowledgment to return, combining propagation delay, processing delay, and queuing delay twice.

Q187. Throughput differs from bandwidth in that:

A) Throughput always equals bandwidth

B) Throughput is the actual data transfer rate achieved, accounting for overhead and losses

C) Throughput only applies to UDP

D) Bandwidth is measured in packets; throughput in bits

✔ Correct Answer: B) Throughput is the actual data transfer rate achieved, accounting for overhead and losses

Explanation: Bandwidth is the theoretical maximum capacity; throughput is the actual usable rate considering protocol overhead, congestion, errors, and retransmissions.

Q188. A network has 1% packet loss. How does this affect a TCP bulk transfer?

A) No impact since TCP recovers automatically

B) Significant throughput reduction due to retransmissions and congestion window reductions

C) Only affects UDP traffic

D) Doubles the effective throughput

✔ Correct Answer: B) Significant throughput reduction due to retransmissions and congestion window reductions

Explanation: TCP interprets packet loss as congestion and reduces its sending rate (AIMD). Even 1% loss dramatically reduces throughput on high-bandwidth links, as predicted by the Mathis formula.

Q189. What is the bandwidth-delay product (BDP)?

A) The minimum number of hops a packet takes

B) The amount of data 'in flight' in a network at any given time (bandwidth × RTT)

C) The maximum file size that can be transferred

D) The product of packet size and TTL

✔ Correct Answer: B) The amount of data 'in flight' in a network at any given time (bandwidth × RTT)

Explanation: BDP = Bandwidth × RTT represents the volume of data that fills the network 'pipe.' TCP window size must be at least BDP to fully utilize high-bandwidth, high-latency links.

Q190. Propagation delay is primarily determined by:

A) Router processing speed

B) Queue length at intermediate routers

C) The physical distance and medium the signal travels through

D) Packet size

✔ Correct Answer: C) The physical distance and medium the signal travels through

Explanation: Propagation delay = Distance / Speed of light in the medium. It is fixed by physical geography and cannot be reduced by adding bandwidth or faster routers.

Q191. Which tool measures the RTT and path to a destination by sending packets with incrementing TTL values?

A) ping

B) nslookup

C) traceroute/tracert

D) netstat

✔ Correct Answer: C) traceroute/tracert

Explanation: Traceroute sends packets with incrementing TTL values (1, 2, 3, …). Each router decrements TTL to 0 and returns an ICMP Time Exceeded, revealing the hop-by-hop path and per-hop RTT.

Q192. The Five 9s (99.999%) availability target for a network means the allowable downtime per year is approximately:

A) 8.76 hours

B) 52.6 minutes

C) 5.26 minutes

D) 26 minutes

✔ Correct Answer: C) 5.26 minutes

Explanation: 99.999% uptime = 0.001% downtime. 0.001% × 525,600 min/year ≈ 5.26 minutes of allowable downtime per year, requiring highly redundant and resilient systems.

Q193. Serialization delay is:

A) The time for a signal to propagate across a cable

B) The time required to place all bits of a packet onto the transmission medium

C) The delay caused by router CPU processing

D) The time a packet waits in a queue

✔ Correct Answer: B) The time required to place all bits of a packet onto the transmission medium

Explanation: Serialization delay = Packet size (bits) / Link bandwidth (bps). On high-bandwidth links it is negligible; on low-speed links (56 kbps), even small packets introduce measurable delay.

Q194. What does MOS (Mean Opinion Score) measure in VoIP networks?

A) Network throughput in Mbps

B) Subjective voice quality rated on a 1–5 scale

C) Number of concurrent VoIP calls supported

D) Packet loss percentage on a WAN link

✔ Correct Answer: B) Subjective voice quality rated on a 1–5 scale

Explanation: MOS is a standardized measure of voice call quality, scored 1 (bad) to 5 (excellent). It accounts for codec quality, packet loss, latency, and jitter in VoIP deployments.

Q195. Which protocol can be used to measure and monitor network interface statistics like bytes in/out and errors?

A) DNS

B) SNMP with MIB OIDs for interface counters

C) DHCP

D) NTP

✔ Correct Answer: B) SNMP with MIB OIDs for interface counters

Explanation: SNMP polls Management Information Base (MIB) OIDs such as ifInOctets and ifOutErrors on network devices to collect interface statistics for monitoring and troubleshooting.

Q196. A bottleneck link in a network path determines:

A) The RTT of the path

B) The maximum achievable throughput for that end-to-end flow

C) The number of hops to the destination

D) The MTU of the path

✔ Correct Answer: B) The maximum achievable throughput for that end-to-end flow

Explanation: The bottleneck is the slowest link in a network path. No matter how fast other links are, end-to-end throughput cannot exceed the capacity of the bottleneck link.

Q197. PMTUD (Path MTU Discovery) is used to:

A) Discover the maximum transmission unit along a network path to avoid fragmentation

B) Measure the RTT to each hop

C) Find the shortest path to a destination

D) Negotiate TCP window size

✔ Correct Answer: A) Discover the maximum transmission unit along a network path to avoid fragmentation

Explanation: PMTUD sends packets with the DF (Don't Fragment) bit set. If a router must fragment, it drops the packet and sends an ICMP 'Fragmentation Needed' message, allowing the sender to reduce packet size.

Q198. In network monitoring, what does a high CPU utilization on a router indicate?

A) The router is operating normally

B) The router may be struggling to process traffic, potentially causing packet drops and increased latency

C) Bandwidth is underutilized

D) The router's cooling fan is failing

✔ Correct Answer: B) The router may be struggling to process traffic, potentially causing packet drops and increased latency

Explanation: Sustained high CPU on a router can indicate excessive routing protocol churn, DDoS traffic, or insufficient hardware for traffic load, leading to slow path processing and performance degradation.

Q199. What is the purpose of NetFlow in network performance monitoring?

A) To encrypt all traffic between routers

B) To export metadata about IP flows (source/destination, bytes, packets) for traffic analysis and capacity planning

C) To assign IP addresses based on traffic load

D) To perform QoS marking on packets

✔ Correct Answer: B) To export metadata about IP flows (source/destination, bytes, packets) for traffic analysis and capacity planning

Explanation: NetFlow collects and exports IP flow data from routers/switches to a collector, enabling traffic analysis, anomaly detection, security monitoring, and capacity planning.

Q200. A network SLA (Service Level Agreement) typically specifies:

A) The brand of hardware used in the network

B) Guaranteed performance metrics such as uptime, latency, jitter, and packet loss thresholds

C) The IP addressing scheme for the network

D) The number of VLANs permitted on the network

✔ Correct Answer: B) Guaranteed performance metrics such as uptime, latency, jitter, and packet loss thresholds

Explanation: An SLA is a contractual commitment between a service provider and customer defining measurable performance targets (availability %, max latency, jitter, packet loss) and remedies for non-compliance.